Федеральное государственное автономное образовательное учреждение

высшего образования

«Национальный исследовательский университет

«Высшая школа экономики»

Московский институт электроники и математики национального

исследовательского университета «Высшая школа экономики»

Департамент компьютерной инженерии

Курс «Системное проектирование цифровых устройств»

ОТЧЕТ

о выполнении практической работы №3

тема работы: «Практические примеры использования ПЛИС часть 2»

Выполнил:

Щеглев Михаил БИВ225

Грушевский Никита БИВ225

Принял:

Зунин В. В.

# Задание

- Разработать устройствор рассчитывающее пройденное расстояние между двумя точками –округление до целогом начальная точка и конечная фиксируется нажатием одной из кнопок или переключателя№л Результат выводится на 7-сегментный индикаторл

- Разработать устройство, которое подсчитывает количество нажатий одной кнопки мыши. Начало и конец интервала измерения фиксируются нажатием одной из кнопок или переключателя. Результат отображается на 7-сегментном индикаторе.

# Выполнение

Был выполнен запуск караоке машины на основе ядра Nius II. Основное отличие данной реализации он теализации непосретственно на чипе плис заключается в том, что в данном случае вся логика обработки информации описываются высокоуровневым языком програмирования скомпилированым и исполняемым на ядре Nius II.

Далее был выполнены задания по работе с мышкой через интерфейс ps2. В нашем случае были реализованны задания по расчёту манхетоновского растояния пройденного мышкой и и подсчёт количества нажатий на одну из кнопок мыши.

# Вывод

В ходе выполнения работы были получены знания по работе с ядром Nius II, интерфейсом ps2. Данные знания полезны для работы с перефирийными устройствами и автоматизации работы используя ядра вместо непосредственного описания алгоритма на языке верилог, что имеет массу приемуществ так как позволяет использовать высокоуровневые языки программирования.

# Приложение

Приложение 1.

Топлевел для подключения ps2 мыши.

// ============================================================================

// Copyright (c) 2013 by Terasic Technologies Inc.

// ============================================================================

//

// Permission:

//

//   Terasic grants permission to use and modify this code for use

//   in synthesis for all Terasic Development Boards and Altera Development

//   Kits made by Terasic.  Other use of this code, including the selling

//   ,duplication, or modification of any portion is strictly prohibited.

//

// Disclaimer:

//

//   This VHDL/Verilog or C/C++ source code is intended as a design reference

//   which illustrates how these types of functions can be implemented.

//   It is the user's responsibility to verify their design for

//   consistency and functionality through the use of formal

//   verification methods.  Terasic provides no warranty regarding the use

//   or functionality of this code.

//

// ============================================================================

//

//  Terasic Technologies Inc

//  9F., No.176, Sec.2, Gongdao 5th Rd, East Dist, Hsinchu City, 30070. Taiwan

//

//

//                     web: http://www.terasic.com/

//                     email: support@terasic.com

//

// ============================================================================

//Date:  Thu Jul 11 11:26:45 2013

// ============================================================================

//`define ENABLE_HPS

module DE1_SoC_PS2_DEMO(

///////// ADC /////////

inout              ADC_CS_N,

output             ADC_DIN,

input              ADC_DOUT,

output             ADC_SCLK,

///////// AUD /////////

input              AUD_ADCDAT,

inout              AUD_ADCLRCK,

inout              AUD_BCLK,

output             AUD_DACDAT,

inout              AUD_DACLRCK,

output             AUD_XCK,

///////// CLOCK2 /////////

input              CLOCK2_50,

///////// CLOCK3 /////////

input              CLOCK3_50,

///////// CLOCK4 /////////

input              CLOCK4_50,

///////// CLOCK /////////

input              CLOCK_50,

///////// DRAM /////////

output      [12:0] DRAM_ADDR,

output      [1:0]  DRAM_BA,

output             DRAM_CAS_N,

output             DRAM_CKE,

output             DRAM_CLK,

output             DRAM_CS_N,

inout       [15:0] DRAM_DQ,

output             DRAM_LDQM,

output             DRAM_RAS_N,

output             DRAM_UDQM,

output             DRAM_WE_N,

///////// FAN /////////

output             FAN_CTRL,

///////// FPGA /////////

output             FPGA_I2C_SCLK,

inout              FPGA_I2C_SDAT,

///////// GPIO /////////

inout     [35:0]         GPIO_0,

inout     [35:0]         GPIO_1,

///////// HEX0 /////////

output      [6:0]  HEX0,

///////// HEX1 /////////

output      [6:0]  HEX1,

///////// HEX2 /////////

output      [6:0]  HEX2,

///////// HEX3 /////////

output      [6:0]  HEX3,

///////// HEX4 /////////

output reg  [6:0]  HEX4,

///////// HEX5 /////////

output reg  [6:0]  HEX5,

`ifdef ENABLE_HPS

///////// HPS /////////

inout              HPS_CONV_USB_N,

output      [14:0] HPS_DDR3_ADDR,

output      [2:0]  HPS_DDR3_BA,

output             HPS_DDR3_CAS_N,

output             HPS_DDR3_CKE,

output             HPS_DDR3_CK_N,

output             HPS_DDR3_CK_P,

output             HPS_DDR3_CS_N,

output      [3:0]  HPS_DDR3_DM,

inout       [31:0] HPS_DDR3_DQ,

inout       [3:0]  HPS_DDR3_DQS_N,

inout       [3:0]  HPS_DDR3_DQS_P,

output             HPS_DDR3_ODT,

output             HPS_DDR3_RAS_N,

output             HPS_DDR3_RESET_N,

input              HPS_DDR3_RZQ,

output             HPS_DDR3_WE_N,

output             HPS_ENET_GTX_CLK,

inout              HPS_ENET_INT_N,

output             HPS_ENET_MDC,

inout              HPS_ENET_MDIO,

input              HPS_ENET_RX_CLK,

input       [3:0]  HPS_ENET_RX_DATA,

input              HPS_ENET_RX_DV,

output      [3:0]  HPS_ENET_TX_DATA,

output             HPS_ENET_TX_EN,

inout       [3:0]  HPS_FLASH_DATA,

output             HPS_FLASH_DCLK,

output             HPS_FLASH_NCSO,

inout              HPS_GSENSOR_INT,

inout              HPS_I2C1_SCLK,

inout              HPS_I2C1_SDAT,

inout              HPS_I2C2_SCLK,

inout              HPS_I2C2_SDAT,

inout              HPS_I2C_CONTROL,

inout              HPS_KEY,

inout              HPS_LED,

inout              HPS_LTC_GPIO,

output             HPS_SD_CLK,

inout              HPS_SD_CMD,

inout       [3:0]  HPS_SD_DATA,

output             HPS_SPIM_CLK,

input              HPS_SPIM_MISO,

output             HPS_SPIM_MOSI,

inout              HPS_SPIM_SS,

input              HPS_UART_RX,

output             HPS_UART_TX,

input              HPS_USB_CLKOUT,

inout       [7:0]  HPS_USB_DATA,

input              HPS_USB_DIR,

input              HPS_USB_NXT,

output             HPS_USB_STP,

`endif /*ENABLE_HPS*/

///////// IRDA /////////

input              IRDA_RXD,

output             IRDA_TXD,

///////// KEY /////////

input       [3:0]  KEY,

///////// LEDR /////////

output      [9:0]  LEDR,

///////// PS2 /////////

inout              PS2_CLK,

inout              PS2_CLK2,

inout              PS2_DAT,

inout              PS2_DAT2,

///////// SW /////////

input       [9:0]  SW,

///////// TD /////////

input              TD_CLK27,

input      [7:0]  TD_DATA,

input             TD_HS,

output             TD_RESET_N,

input             TD_VS,

///////// VGA /////////

output      [7:0]  VGA_B,

output             VGA_BLANK_N,

output             VGA_CLK,

output      [7:0]  VGA_G,

output             VGA_HS,

output      [7:0]  VGA_R,

output             VGA_SYNC_N,

output             VGA_VS

);

//=======================================================

//  REG/WIRE declarations

//=======================================================

wire HEX0P;

wire HEX1P;

wire HEX2P;

wire HEX3P;

wire HEX4P;

wire HEX5P;

wire action;

//=======================================================

//  Structural coding

//=======================================================

///////////////////////////////////////////

// VGA

assign VGA_BLANK_N = 1'b1;

assign VGA_SYNC_N = 1'b1;

///////////////////////////////////////////

// TV

assign TD_RESET_N = TD_VS;

assign action = TD_HS & TD_CLK27 & (TD_DATA == 8'hff);

wire [7:0] x, y;

reg [7:0] x_regged, y_regged;

///////////////////////////////////////////

//ps2

//////////////////////////////////////

///intantiation

ps2 U1(.iSTART(KEY[0]),  //press the button for transmitting instrucions to device;

.iRST_n(KEY[1]),  //global reset signal;

.iCLK_50(CLOCK_50),  //clock source;

.PS2_CLK(PS2_CLK), //ps2_clock signal inout;

.PS2_DAT(PS2_DAT), //ps2_data  signal inout;

.oLEFBUT(LEDR[0]),  //left button press display;

.oRIGBUT(LEDR[1]),  //right button press display;

.oMIDBUT(LEDR[2]),  //middle button press display;

.oX_MOV1(HEX0),  //lower SEG of mouse displacement display for X axis.

.oX_MOV2(HEX1),  //higher SEG of mouse displacement display for X axis.

.oY_MOV1(HEX2),  //lower SEG of mouse displacement display for Y axis.

.oY_MOV2(HEX3),

.x_latch(x),

.y_latch(y)); //higher SEG of mouse displacement display for Y axis.

reg [7:0] distance, distance_next;

always @(posedge CLOCK_50 or negedge KEY[1]) begin

if (!KEY[1]) begin

x_regged <= 0;

y_regged <= 0;

distance <= 0;

end

else begin

if (LEDR[0]) begin

x_regged <= x;

y_regged <= y;

distance <= distance_next;

end

end

end

always @(*) begin

distance_next = 0;

if (x_regged > x) begin

distance_next = distance_next + x_regged - x;

end

else begin

distance_next = distance_next - x_regged + x;

end

if (y_regged > y) begin

distance_next = distance_next + y_regged - y;

end

else begin

distance_next = distance_next - y_regged + y;

end

end

reg [7:0] counter;

reg catch_key;

always @(posedge LEDR[0] or negedge KEY[3]) begin

if (!KEY[3]) begin

counter <= 0;

catch_key <= 0;

end

else begin

catch_key <= 1;

counter <= counter + 1;

end

end

reg lc, rc, mc, x_sign, y_sign;

reg [7:0] x_regged_2, y_regged_2, dist_x, dist_y;

reg [6:0] HEX4_l, HEX5_l;

wire [6:0] HEX4_w, HEX5_w;

reg [3:0] buf1, buf2;

always @(negedge KEY[3]) begin

if(catch_key) begin

buf1 <= counter[3:0];

buf2 <= counter[7:4];

end

end

SEG7_LUT lut4 (HEX4_w, buf1);

SEG7_LUT lut5 (HEX5_w, buf2);

always @(*) begin

if (!SW[0]) begin

HEX4 = HEX4_w;

HEX5 = HEX5_w;

end

else begin

HEX4 = HEX4_l;

HEX5 = HEX5_l;

end

end

endmodule

Приложение 2.

Модуль для работы с мышью по интерфейсу PS2.

//

// Permission:

//

//   Terasic grants permission to use and modify this code for use

//   in synthesis for all Terasic Development Boards and Altera Development

//   Kits made by Terasic.  Other use of this code, including the selling

//   ,duplication, or modification of any portion is strictly prohibited.

//

// Disclaimer:

//

//   This VHDL/Verilog or C/C++ source code is intended as a design reference

//   which illustrates how these types of functions can be implemented.

//   It is the user's responsibility to verify their design for

//   consistency and functionality through the use of formal

//   verification methods.  Terasic provides no warranty regarding the use

//   or functionality of this code.

//

// --------------------------------------------------------------------

//

//                     Terasic Technologies Inc

//                     356 Fu-Shin E. Rd Sec. 1. JhuBei City,

//                     HsinChu County, Taiwan

//                     302

//

//                     web: http://www.terasic.com/

//                     email: support@terasic.com

//

// --------------------------------------------------------------------

//

// Major Functions:	DE2_115_PS2 Mouse Controller

//

// --------------------------------------------------------------------

//

// Revision History :

// --------------------------------------------------------------------

//   Ver  :| Author                    :| Mod. Date :| Changes Made:

//   V1.0 :| Johnny FAN,HdHuang        :| 05/16/10  :| Initial Revision

// --------------------------------------------------------------------

module ps2(

iSTART,   //press the button for transmitting instrucions to device;

iRST_n,   //FSM reset signal;

iCLK_50,  //clock source;

PS2_CLK,  //ps2_clock signal inout;

PS2_DAT,  //ps2_data  signal inout;

oLEFBUT,  //left button press display;

oRIGBUT,  //right button press display;

oMIDBUT,  //middle button press display;

oX_MOV1,  //lower SEG of mouse displacement display for X axis.

oX_MOV2,  //higher SEG of mouse displacement display for X axis.

oY_MOV1,  //lower SEG of mouse displacement display for Y axis.

oY_MOV2,   //higher SEG of mouse displacement display for Y axis.

x_latch,

y_latch

);

//interface;

//=======================================================

//  PORT declarations

//=======================================================

input iSTART;

input iRST_n;

input iCLK_50;

inout PS2_CLK;

inout PS2_DAT;

output oLEFBUT;

output oRIGBUT;

output oMIDBUT;

output [6:0] oX_MOV1;

output [6:0] oX_MOV2;

output [6:0] oY_MOV1;

output [6:0] oY_MOV2;

output reg [7:0] x_latch;

output reg [7:0] y_latch;

//instantiation

SEG7_LUT U1(.oSEG(oX_MOV1),.iDIG(x_latch[3:0]));

SEG7_LUT U2(.oSEG(oX_MOV2),.iDIG(x_latch[7:4]));

SEG7_LUT U3(.oSEG(oY_MOV1),.iDIG(y_latch[3:0]));

SEG7_LUT U4(.oSEG(oY_MOV2),.iDIG(y_latch[7:4]));

//instruction define, users can charge the instruction byte here for other purpose according to ps/2 mouse datasheet.

//the MSB is of parity check bit, that's when there are odd number of 1's with data bits, it's value is '0',otherwise it's '1' instead.

parameter enable_byte =9'b011110100;

//=======================================================

//  REG/WIRE declarations

//=======================================================

reg [1:0] cur_state,nex_state;

reg ce,de;

reg [3:0] byte_cnt,delay;

reg [5:0] ct;

reg [7:0] cnt;

reg [8:0] clk_div;

reg [9:0] dout_reg;

reg [32:0] shift_reg;

reg       leflatch,riglatch,midlatch;

reg       ps2_clk_in,ps2_clk_syn1,ps2_dat_in,ps2_dat_syn1;

wire      clk,ps2_dat_syn0,ps2_clk_syn0,ps2_dat_out,ps2_clk_out,flag;

//=======================================================

//  PARAMETER declarations

//=======================================================

//state define

parameter listen =2'b00,

pullclk=2'b01,

pulldat=2'b10,

trans  =2'b11;

//=======================================================

//  Structural coding

//=======================================================

//clk division, derive a 97.65625KHz clock from the 50MHz source;

always@(posedge iCLK_50)

begin

clk_div <= clk_div+1;

end

assign clk = clk_div[8];

//tristate output control for PS2_DAT and PS2_CLK;

assign PS2_CLK = ce?ps2_clk_out:1'bZ;

assign PS2_DAT = de?ps2_dat_out:1'bZ;

assign ps2_clk_out = 1'b0;

assign ps2_dat_out = dout_reg[0];

assign ps2_clk_syn0 = ce?1'b1:PS2_CLK;

assign ps2_dat_syn0 = de?1'b1:PS2_DAT;

//

assign oLEFBUT = leflatch;

assign oRIGBUT = riglatch;

assign oMIDBUT = midlatch;

//multi-clock region simple synchronization

always@(posedge clk)

begin

ps2_clk_syn1 <= ps2_clk_syn0;

ps2_clk_in   <= ps2_clk_syn1;

ps2_dat_syn1 <= ps2_dat_syn0;

ps2_dat_in   <= ps2_dat_syn1;

end

//FSM shift

always@(*)

begin

case(cur_state)

listen  :begin

if ((!iSTART) && (cnt == 8'b11111111))

nex_state = pullclk;

else

nex_state = listen;

ce = 1'b0;

de = 1'b0;

end

pullclk :begin

if (delay == 4'b1100)

nex_state = pulldat;

else

nex_state = pullclk;

ce = 1'b1;

de = 1'b0;

end

pulldat :begin

nex_state = trans;

ce = 1'b1;

de = 1'b1;

end

trans   :begin

if  (byte_cnt == 4'b1010)

nex_state = listen;

else

nex_state = trans;

ce = 1'b0;

de = 1'b1;

end

default :    nex_state = listen;

endcase

end

//idle counter

always@(posedge clk)

begin

if ({ps2_clk_in,ps2_dat_in} == 2'b11)

begin

cnt <= cnt+1;

end

else begin

cnt <= 8'd0;

end

end

//periodically reset ct; ct counts the received data length;

assign flag = (cnt == 8'hff)?1:0;

always@(posedge ps2_clk_in,posedge flag)

begin

if (flag)

ct <= 6'b000000;

else

ct <= ct+1;

end

//latch data from shift_reg;outputs is of 2's complement;

//Please treat the cnt value here with caution, otherwise wrong data will be latched.

always@(posedge clk,negedge iRST_n)

begin

if (!iRST_n)

begin

leflatch <= 1'b0;

riglatch <= 1'b0;

midlatch <= 1'b0;

x_latch  <= 8'd0;

y_latch  <= 8'd0;

end

else if (cnt == 8'b00011110 && (ct[5] == 1'b1 || ct[4] == 1'b1))

begin

leflatch <= shift_reg[1];

riglatch <= shift_reg[2];

midlatch <= shift_reg[3];

x_latch  <= x_latch+shift_reg[19 : 12];

y_latch  <= y_latch+shift_reg[30 : 23];

end

end

//pull ps2_clk low for 100us before transmit starts;

always@(posedge clk)

begin

if (cur_state == pullclk)

delay <= delay+1;

else

delay <= 4'b0000;

end

//transmit data to ps2 device;eg. 0xF4

always@(negedge ps2_clk_in)

begin

if (cur_state == trans)

dout_reg <= {1'b0,dout_reg[9:1]};

else

dout_reg <= {enable_byte,1'b0};

end

//transmit byte length counter

always@(negedge ps2_clk_in)

begin

if (cur_state == trans)

byte_cnt <= byte_cnt+1;

else

byte_cnt <= 4'b0000;

end

//receive data from ps2 device;

always@(negedge ps2_clk_in)

begin

if (cur_state == listen)

shift_reg <= {ps2_dat_in,shift_reg[32:1]};

end

//FSM movement

always@(posedge clk,negedge iRST_n)

begin

if (!iRST_n)

cur_state <= listen;

else

cur_state <= nex_state;

end

endmodule