Федеральное государственное автономное образовательное учреждение

высшего образования

«Национальный исследовательский университет

«Высшая школа экономики»

Московский институт электроники и математики национального

исследовательского университета «Высшая школа экономики»

Департамент компьютерной инженерии

Курс «Системное проектирование цифровых устройств»

ОТЧЕТ

о выполнении практической работы №1

тема работы: «Обработка звука на ПЛИС»

Выполнил:

Щеглев Михаил БИВ225

Грушевский Никита БИВ225

Принял:

Зунин В. В.

# Задание

# В соответствии с мануалом и используя дополнительные материалы создать проект, в котором сигнал с линейного входа от микрофона подаётся на линейный выход на динамики.

- Добавить управление от кнопки, убирающее один из каналов или меняющее каналы местами.

- Разработать генератор шума.

- Выполнить прототипирование проекта.

- Разработать индикацию звука на семисегментном индикаторе.

- Реализовать диктофон.

# Выполнение

## Вывод звука на динамик

Чтобы звук воспроизводился на динамике, главный модуль необходимо направить обработанный сигнал с микрофона на выход и определить сигнал чтения и записи по готовности кодека:

assign writedata_left =  left_out;

assign writedata_right = right_out;

assign read = read_ready;

assign write = write_ready;

## 2.2 — 2.3. Перестановка каналов и генератор шума

Чтобыпоменять местами каналы нужно поменять местами входные каналы при передаче их в выходные (по которому на левый канал динамика будут подаваться сэмплы с правого канала микрофона и наоборот). Для ддобавления шума необходимо провести логическое или с сгенерированным сигналом шума (Листинг 1).

Листинг 1 — Код ...

if (switch_channel) begin

left_out = noise_enable ? (readdata_left | noise) : readdata_left;

right_out = noise_enable ? (readdata_right | noise) : readdata_right;

end else begin

right_out = noise_enable ? (readdata_left | noise) : readdata_left;

left_out = noise_enable ? (readdata_right | noise) : readdata_right;

end

## 2.4. Индикация.

На светодиодную ленту выводиться информация о статусе работы устройства в данный момент:

assign LEDR[0] = switch_channel;

assign LEDR[1] = noise_enable;

assign LEDR[2] = record_button;

assign LEDR[4] = playing;

assign LEDR[5] = recording;

assign LEDR[6] = record_button;

assign LEDR[7] = stop_record_button;

assign LEDR[8] = play_button;

assign LEDR[9] = stop_play_button;

## 2.5. Диктафон.

Диктафон по сигналу старта записи записывает в память данные с входа до тех пор, пока не будет подан сигнал остановки записи или пока не будет заполнена вся доступная память. Воспроизведение работает по аналогичному принципу. Код диктофона приведён в приложении 1.

# Вывод

В ходе изучения мануала Basic Digital Signal Processing была изучены теория цифровой обработки звука и конкретно методика работы со звуком на ПЛИС. Был произведен вывод звука с микрофона на динамик (наушники), разработан и применен модуль наложения шума и диктофон.

# Приложение

Приложение 1.

Диктофон

module dictaphone #(

parameter MEMORY_SIZE = 4096  // Размер памяти (можно настроить)

)(

input logic clk,

input logic reset,

input logic record_btn,

input logic stop_record_btn,

input logic play_btn,

input logic stop_play_btn,

input logic [23:0] audio_in_left,

input logic [23:0] audio_in_right,

input logic read_ready,

input logic write_ready,

output logic playing,

output logic recording,

output logic audio_out_left,

output logic audio_out_right

);

// Параметры памяти

localparam MEMORY_WIDTH = $clog2(MEMORY_SIZE);

// Память для записи

logic [23:0] memory_left [MEMORY_SIZE];

logic [23:0] memory_right [MEMORY_SIZE];

// Указатели записи/воспроизведения

logic [MEMORY_WIDTH-1:0] write_pointer;

logic [MEMORY_WIDTH-1:0] read_pointer;

logic [MEMORY_WIDTH-1:0] record_length;

// Состояния конечного автомата

enum {IDLE, RECORDING_STATE, PLAYING_STATE} state, next_state;

assign recording = state == RECORDING_STATE;

assign playing   = state == PLAYING_STATE;

always_ff @(posedge clk or posedge reset) begin

if (reset) begin

state <= IDLE;

end else begin

state <= next_state;

end

end

always_comb begin

case (state)

IDLE: begin

next_state = IDLE;

if (record_btn && !stop_record_btn) begin

next_state = RECORDING_STATE;

end else if (play_btn && !stop_play_btn) begin

next_state = PLAYING_STATE;

end

end

RECORDING_STATE: begin

next_state = RECORDING_STATE;

if (stop_record_btn || write_pointer >= (MEMORY_SIZE-1'b1)) begin

next_state = IDLE;

end

end

PLAYING_STATE: begin

next_state = PLAYING_STATE;

if (stop_play_btn) begin

next_state = IDLE;

end

end

endcase

end

// Конечный автомат управления

always @(posedge clk or posedge reset) begin

if (reset) begin

write_pointer   <= '0;

read_pointer    <= '0;

audio_out_left  <= '0;

audio_out_right <= '0;

end else begin

audio_out_left  <= '0;

audio_out_right <= '0;

case (state)

IDLE: begin

case(next_state)

RECORDING_STATE: begin

write_pointer <= '0;

end

PLAYING_STATE: begin

read_pointer <= '0;

end

default:;

endcase

end

RECORDING_STATE: begin

if (read_ready && write_pointer < MEMORY_SIZE) begin

// Запись в память

memory_left[write_pointer] <= audio_in_left;

memory_right[write_pointer] <= audio_in_right;

write_pointer <= write_pointer + 1'b1;

end

end

PLAYING_STATE: begin

if (write_ready && read_pointer < write_pointer) begin

// Воспроизведение из памяти

audio_out_left  <= memory_left[read_pointer];

audio_out_right <= memory_right[read_pointer];

read_pointer <= read_pointer + 1;

end else if (read_pointer >= write_pointer) begin

// Циклическое воспроизведение

read_pointer <= '0;

end

end

endcase

end

end

endmodule

Приложение 2.

Оснновной код.

module part1 (CLOCK_50, CLOCK2_50, KEY, FPGA_I2C_SCLK, FPGA_I2C_SDAT, AUD_XCK,

AUD_DACLRCK, AUD_ADCLRCK, AUD_BCLK, AUD_ADCDAT, AUD_DACDAT, SW, LEDR);

input CLOCK_50, CLOCK2_50;

input [3:0] KEY;

output FPGA_I2C_SCLK;

inout FPGA_I2C_SDAT;

output AUD_XCK;

input AUD_DACLRCK, AUD_ADCLRCK, AUD_BCLK;

input AUD_ADCDAT;

input [9:0] SW;

output AUD_DACDAT;

output [9:0] LEDR;

// Основные сигналы аудио

logic read_ready, write_ready, read, write;

logic [23:0] readdata_left, readdata_right;

logic [23:0] writedata_left, writedata_right;

// Сигналы управления

wire reset = ~KEY[0];

wire switch_channel = SW[0];

wire noise_enable = SW[1];

wire record_button = SW[2];

wire stop_record_button = SW[3];

wire play_button = SW[4];

wire stop_play_button = SW[5];

wire noise_enable_sw = SW[9];

assign LEDR[0] = switch_channel;

assign LEDR[1] = noise_enable;

assign LEDR[2] = record_button;

assign LEDR[4] = playing;

assign LEDR[5] = recording;

assign LEDR[6] = record_button;

assign LEDR[7] = stop_record_button;

assign LEDR[8] = play_button;

assign LEDR[9] = stop_play_button;

// Сигналы диктофона

logic [23:0] noise;

logic [23:0] recorded_left, recorded_right;

logic recording, playing;

noise_gen ng(CLOCK_50,noise_enable_sw, noise);

reg [23:0] left_out;

reg [23:0] right_out;

always_comb begin

if (playing) begin

// Режим воспроизведения записи

left_out = recorded_left;

right_out = recorded_right;

end else if (recording) begin

// Режим записи - выводим входной сигнал

left_out = readdata_left;

right_out = readdata_right;

end else begin

// Обычный режим с эффектами

if (switch_channel) begin

left_out = noise_enable ? (readdata_left | noise) : readdata_left;

right_out = noise_enable ? (readdata_right | noise) : readdata_right;

end else begin

right_out = noise_enable ? (readdata_left | noise) : readdata_left;

left_out = noise_enable ? (readdata_right | noise) : readdata_right;

end

end

end

assign writedata_left =  left_out;

assign writedata_right = right_out;

assign read = read_ready;

assign write = write_ready;

dictaphone #(.MEMORY_SIZE(15000)

) dictaphone_unit(

.clk(FPGA_I2C_SCLK),

.reset(reset),

.record_btn(record_button),

.stop_record_btn(stop_record_button),

.play_btn(play_button),

.stop_play_btn(stop_play_button),

.audio_in_left(readdata_left),

.audio_in_right(readdata_right),

.read_ready(read_ready),

.write_ready(write_ready),

.audio_out_left(recorded_left),

.audio_out_right(recorded_right),

.recording(recording),

.playing(playing),

);

clock_generator my_clock_gen(

// inputs

CLOCK_50,

reset,

// outputs

AUD_XCK

);

audio_and_video_config cfg(

// Inputs

CLOCK_50,

reset,

// Bidirectionals

FPGA_I2C_SDAT,

FPGA_I2C_SCLK

);

audio_codec codec(

// Inputs

CLOCK_50,

reset,

read,	write,

writedata_left, writedata_right,

AUD_ADCDAT,

// Bidirectionals

AUD_BCLK,

AUD_ADCLRCK,

AUD_DACLRCK,

// Outputs

read_ready, write_ready,

readdata_left, readdata_right,

AUD_DACDAT

);

endmodule